EMERGENCY CHECKLIST							FORM	E04

GROUNDING

| Immediate Actions | DONE |
| --- | --- |
| a Call the Master. |  |
| b Check for personal injuries, damages to ship and cargo. Check for possible leakages, take soundings of tank and bilges. (SALVAGE OF OWN SHIP PLAN to be used in case of savage). |  |
| c Crew stand-by. |  |
| d Prepare life-boats and life-saving equipment. |  |
| e Keep the radio station stand-by - with current and updated position available. |  |
| f Show applicable signal from the International Code of Signals Manual. (VHF could also be used to indicate distress). |  |
| g Lay down the exact position on the sea chart indicating the heading. Fix time. |  |
| h Take soundings around the ship. Sketch the ship with draft before and after the grounding indicating soundings taken as well as the bottom texture. Record the time soundings taken. |  |
| i Check tide table for HW and LW times and range. |  |
| j Check weather forecast, wind and current, direction and force. |  |
| k Consider the risk of heavy waves, strong current or flood, and if filling of empty tanks is needed in order to prevent the ship from drifting higher up or off the grounding spot. |  |
| l Judge whether refloat attempts should be made or not based on above information and trim and stability calculations as well as possible damages to propeller and rudder, which may cause manoeuvring problem once deflated. Consider the increased risk of oil spill. |  |
| m Do not underestimate the risk of oil spill. Therefore, always make preparations for oil pollution preventions. (Ref: SOPEP Manual). Some state authorities might require clean-up contractors stand-by before any refloat attempts are allowed. |  |
| n It could be wise to wait with the refloat attempt until the ships position on the grounding spot, possible damages to the propeller, rudder. main engine, steering gear and/or lubricating oil tanks are thoroughly examined, as well as the arrival of the Clean-Up Contractors. |  |
| o Take necessary actions to minimise further damages to personnel, environment and ships. (SOPEP Manual to be used in case of oil spill) |  |
| Then: p Report to the Company by the fastest and most efficient way. Keep them continuously informed. q Enter continuously any actions taken in ship's log book. |  |