| EMBARK/DISEMBARK PILOT CHECKLIST |  |  |  |  |
| --- | --- | --- | --- | --- |
| SHIP NAME: |  | PORT: |  |  |
| This checklist is to be completed prior to embarking and disembarking a pilot. It is to be completed by the OOW and a Log entry made. |  |  |  | INITIALS* |
| Advise the following of ETA/ETD: |  |  |  |  |
| Master |  |  |  |  |
| Engine Room |  |  |  |  |
| Pilot Station |  |  |  |  |
| Has the Ship to Shore Master/Pilot Exchange been sent? |  |  |  |  |
| Has the Shore to Ship Master/Pilot Exchange been sent and a completed copy returned? |  |  |  |  |
| Agree which side the pilot will embark/disembark. |  |  |  |  |
| RIGGING THE PILOT LADDER IN COMPLIANCE WITH THE I.M.O. AND I.M.P.A. RECOMMENDATIONS. |  |  |  |  |
| Freeboard 9 m or less use the pilot ladder? |  |  |  |  |
| Freeboard 9 m or more use the pilot ladder and accommodation ladder combination? |  |  |  |  |
| Bulwark ladder secured to ship? |  |  |  |  |
| Lifebuoy with self igniting light prepared by ladder? |  |  |  |  |
| Heaving line available/ready by pilot ladder? |  |  |  |  |
| No tripping lines attached to ladder? |  |  |  |  |
| Side ropes minimum diameter of 18 mm? |  |  |  |  |
| Is the pilot ladder and boarding area clean and free of cargo residue and obstructions? |  |  |  |  |
| Has a deck officer been nominated to meet the pilot and conduct him to/from the bridge? |  |  |  |  |
| Have the following documents been prepared and made ready for the pilot’s boarding? |  |  |  |  |
| Pilot Card (This is to be handed to and discussed with the pilot on boarding? |  |  |  |  |
| Master/Pilot Exchange of Essential Information. (This is to be completed as soon as the pilot boards and prior to commencing the pilotage operation) |  |  |  |  |
| OOW’s SIGNATURE: |  |  | DATE: |  |

* Initialling confirms compliance.  If not applicable enter N/A.  Any additional items to be entered in blanks.